CISCO IOS

All cisco devices (routers, switches, firewalls,….) uses the same operating system called IOS. Devices use the same basic IOS command but they differs by command sets available. Some commands are available only on some decises while disabled in other devices.

Cisco IOS command modes are hierarchical, defining which commands are available for managing devices. The primary modes are User EXEC (>), Privileged EXEC (#), and Global Configuration ((config)#), which branch into sub-configuration modes (e.g., interface, line) to change specific settings.

www.cisco.com +2

Key Command Modes & Prompts:

- User EXEC Mode (Router>): Limited viewing capabilities; first mode upon login.

- Privileged EXEC Mode (Router#): Accessed via enable; allows viewing, saving, and debugging configuration.

- Global Configuration Mode (Router(config)#): Accessed via configure terminal; changes apply to the entire device.

- Interface Configuration Mode (Router(config-if)#): Used to configure specific port parameters (e.g., interface gigabitethernet0/1).

- Line Configuration Mode (Router(config-line)#): Used to set up terminal lines (e.g., line vty 0 4).

- Router Configuration Mode (Router(config-router)#): Used for routing protocol configuration (e.g., router ospf 1).

www.cisco.com +4

Navigation Commands:

- enable: Moves from User to Privileged EXEC mode.

- configure terminal: Moves to Global Configuration mode.

- exit: Returns to the previous mode.

- end (or Ctrl+Z): Exits to Privileged EXEC mode from any nested mode.

- ?: Context-sensitive help to see available commands.

Basic cisco IOS commands

Here are the basic Cisco IOS commands you asked for:

1️⃣ To set the hostname

enable
configure terminal
hostname Router1

Example result:

Router1(config)#

2️⃣ To configure an Ethernet interface

Example configuring FastEthernet0/0 with an IP address:

enable
configure terminal
interface fastethernet0/0
ip address 192.168.1.1 255.255.255.0
no shutdown
exit

Explanation:

- interface fastethernet0/0 → enter interface configuration mode

- ip address → assign IP address and subnet mask

- no shutdown → activate the interface

✅ Example full configuration

enable
configure terminal
hostname Router1
interface fastethernet0/0
ip address 192.168.1.1 255.255.255.0
no shutdown
end

3- to configure serial interface

3️⃣ To configure a Serial interface

Example configuring Serial0/0/0:

enable
configure terminal
interface serial0/0/0
ip address 10.0.0.1 255.255.255.252
clock rate 64000
no shutdown
exit

Explanation:

- interface serial0/0/0 → enter the serial interface configuration mode

- ip address → assign IP address and subnet mask

- clock rate 64000 → set clock rate (only on the DCE side)

- no shutdown → enable the interface

✅ Example full configuration

enable
configure terminal
hostname Router1
interface serial0/0/0
ip address 10.0.0.1 255.255.255.252
clock rate 64000
no shutdown
end

4️⃣ To build a Static Routing Table

Use the command ip route in global configuration mode.

Syntax

ip route [destination-network] [subnet-mask] [next-hop-ip]

Example

enable
configure terminal
ip route 192.168.2.0 255.255.255.0 10.0.0.2

Explanation:

- 192.168.2.0 → destination network

- 255.255.255.0 → subnet mask

- 10.0.0.2 → next-hop router IP

5️⃣ To enable RIP Routing(Dynamic routing)

enable
configure terminal
router rip
version 2
network 192.168.1.0
network 10.0.0.0
no auto-summary

Explanation:

- router rip → start RIP configuration

- version 2 → use RIP version 2

- network → specify networks to advertise

- no auto-summary → disable automatic summarization

✅ Example complete configuration

enable
configure terminal
router rip
version 2
network 192.168.1.0
network 10.0.0.0
no auto-summary
end

🔐 Configuring Access Lists on Cisco Routers

1️⃣ Enter Configuration Mode

First enter privileged mode and global configuration mode.

enable
configure terminal

2️⃣ Configure a Standard Access List

Standard ACLs filter traffic based only on the source IP address.

Syntax

access-list [number] permit|deny [source-ip] [wildcard-mask]

Example

Allow network 192.168.1.0 and deny all others.

access-list 10 permit 192.168.1.0 0.0.0.255

3️⃣ Apply the Access List to an Interface

You must apply the ACL to an interface inbound or outbound.

Syntax

interface [interface-name]
ip access-group [ACL-number] in

Example

interface fastethernet0/0
ip access-group 10 in
exit

4️⃣ Configure an Extended Access List

Extended ACLs filter by:

- Source IP

- Destination IP

- Protocol

- Port number

Syntax

access-list [100-199] permit|deny protocol source destination

Example

Allow HTTP traffic only.

access-list 100 permit tcp 192.168.1.0 0.0.0.255 any eq 80

5️⃣ Apply Extended ACL to an Interface

Example:

interface fastethernet0/1
ip access-group 100 out

6️⃣ Verify Access Lists

Check configured ACLs:

show access-lists

Check interface ACL assignment:

show running-config

Check interface status:

show ip interface

📘 Example Complete Configuration

enable
configure terminal

access-list 10 permit 192.168.1.0 0.0.0.255

interface fastethernet0/0
ip access-group 10 in

end

✅ Important Notes

- Standard ACL numbers: 1 – 99

- Extended ACL numbers: 100 – 199

- ACLs are processed top to bottom

- At the end there is an implicit deny all