# Cisco ASA Firewall Lab with Inside, Outside, and DMZ1 Networks

This document provides a complete laboratory guide for configuring a Cisco ASA firewall using Inside, Outside, and DMZ1 security zones. The DMZ network hosts a public web server. The lab includes DHCP services, NAT, ACL filtering, web filtering, and IDS/IPS integration. Each configuration command is explained in detail to help students understand the purpose and behavior of the configuration.

## 1. Network Topology

The topology consists of three main security zones. The Inside network contains trusted users. The Outside network simulates the Internet. The DMZ network hosts public-facing services such as a web server.

## 2. IP Addressing Plan

| Zone | Device | Interface | IP Address |
| --- | --- | --- | --- |
| Outside | Internet Router | G0/0 | 200.1.1.1/24 |
| Outside | Cisco ASA | G0/0 | 200.1.1.2/24 |
| Inside | Cisco ASA | G0/1 | 192.168.10.1/24 |
| Inside | PC1 and PC2 | DHCP | 192.168.10.50+ |
| DMZ1 | Cisco ASA | G0/2 | 172.16.1.1/24 |
| DMZ1 | Web Server | NIC | 172.16.1.100/24 |

## 3. Packet Tracer Topology Construction Guide

Open Cisco Packet Tracer and create a new project.

Add one Cisco ASA firewall to the workspace.

Add one router to simulate the Internet connection.

Add two switches to represent the Inside and DMZ networks.

Add two PCs to the Inside switch.

Add one web server to the DMZ switch.

Add an optional IDS/IPS sensor.

Connect all devices using copper straight-through cables.

Assign IP addresses according to the addressing table.

Enable HTTP and HTTPS services on the web server.

## 4. ASA Basic Configuration

enable

Enters privileged EXEC mode to allow administrative configuration.

configure terminal

Enters global configuration mode.

hostname ASA-FW

Changes the firewall hostname to ASA-FW.

interface gigabitEthernet0/0

Selects the outside interface connected to the Internet.

nameif outside

Assigns the logical name 'outside' to the interface.

security-level 0

Sets the security level to 0, representing the least trusted network.

ip address 200.1.1.2 255.255.255.0

Assigns the IP address to the outside interface.

no shutdown

Enables the interface.

interface gigabitEthernet0/1

Selects the inside interface.

nameif inside

Assigns the logical name 'inside'.

security-level 100

Sets the highest trust level for internal users.

ip address 192.168.10.1 255.255.255.0

Assigns the gateway IP address for Inside users.

interface gigabitEthernet0/2

Selects the DMZ interface.

nameif dmz1

Assigns the logical name dmz1.

security-level 50

Assigns a medium trust level to the DMZ.

ip address 172.16.1.1 255.255.255.0

Assigns the gateway IP address for the DMZ network.

## 5. DHCP Configuration

dhcpd address 192.168.10.50-192.168.10.100 inside

Creates a DHCP pool for Inside users.

dhcpd dns 208.67.222.222 208.67.220.220 interface inside

Assigns DNS servers to DHCP clients.

dhcpd domain lab.local

Assigns a domain name to DHCP clients.

dhcpd enable inside

Enables the DHCP service on the Inside interface.

## 6. NAT Configuration

object network INSIDE-NET

Creates a network object for the Inside network.

subnet 192.168.10.0 255.255.255.0

Defines the subnet belonging to the Inside network object.

nat (inside,outside) dynamic interface

Configures PAT so Inside users can access the Internet using the ASA outside IP.

object network WEB-SERVER

Creates a network object for the DMZ web server.

host 172.16.1.100

Defines the IP address of the DMZ web server.

nat (dmz1,outside) static 200.1.1.100

Maps the internal web server to a public IP address.

## 7. ACL Configuration

access-list OUTSIDE-IN extended permit tcp any host 172.16.1.100 eq 80

Allows HTTP traffic from the Internet to the DMZ web server.

access-list OUTSIDE-IN extended permit tcp any host 172.16.1.100 eq 443

Allows HTTPS traffic from the Internet to the DMZ web server.

access-list OUTSIDE-IN extended deny ip any 192.168.10.0 255.255.255.0

Blocks all direct access attempts to the Inside network.

access-list OUTSIDE-IN extended deny tcp any any eq 23

Blocks Telnet traffic for security purposes.

access-list OUTSIDE-IN extended deny ip any any

Blocks all remaining traffic not explicitly permitted.

access-group OUTSIDE-IN in interface outside

Applies the ACL to inbound traffic on the outside interface.

## 8. Web Filtering Configuration

access-list WEB-FILTER extended deny tcp any host 157.240.0.0 eq 80

Blocks HTTP access to selected websites.

access-list WEB-FILTER extended deny tcp any host 157.240.0.0 eq 443

Blocks HTTPS access to selected websites.

access-list WEB-FILTER extended permit tcp any any eq 80

Allows general HTTP traffic.

access-list WEB-FILTER extended permit tcp any any eq 443

Allows general HTTPS traffic.

## 9. IDS/IPS Integration

The IDS/IPS device monitors network traffic for malicious activities such as port scanning, SQL injection attempts, malware communication, and ICMP flooding. The IDS/IPS can be deployed inline or passively depending on the organization’s security policy.

## 10. Suggested Verification Commands

show interface ip brief

Displays the status and IP addresses of interfaces.

show access-list

Displays ACL entries and packet counters.

show nat

Displays configured NAT rules.

show conn

Displays active connections through the firewall.

show dhcpd binding

Displays DHCP-assigned addresses.

## 11. Security Testing Scenarios

Verify that Inside PCs receive IP addresses automatically.

Verify that Inside users can browse the Internet.

Verify that HTTP and HTTPS access to the DMZ web server is successful.

Verify that Telnet traffic is blocked.

Verify that direct access from the Internet to the Inside network is denied.

Verify that the IDS/IPS sensor detects suspicious traffic.

## 12. Conclusion

This laboratory demonstrates the implementation of a secure Cisco ASA firewall architecture using multiple security zones. The lab combines firewall filtering, NAT, DHCP, web filtering, and IDS/IPS integration to provide a realistic enterprise security deployment.